# METHOD OF STATEMENT

Date: 16th Apr 2021

Description of work

To unload 3 cases from 1 x 40ft GP container, uncrate, hoist up onto lorry chassis, reverse lorry into factory premise, using customer overhead crane to

Machine Spec

2 unit SC-36 CNC Lathe

Wt: 17, 000 kg each

1 unit SC-25 CNC Lathe

Wt: 9, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

1st unit – 28/03/2020

2nd unit – ETA 05/04/20

3rd unit – TBA

Location of Operation

#### No 80 Tuas West Drive

Dril-Quip Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

Customer 20 ton overhead crane

Wire sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

- Lorry crane to park at designated location (back to back with container)

- Pull out machine base from container and load onto lorry chassis

- Transport lorry into factory production area

- Using customer overhead crane to unload from lorry chassis

- Shift and position main table onto foundation

- Unload balanced item and put aside

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175