# METHOD OF STATEMENT

Date: 28th May 2025

Description of Work

To shift out 2 units machine from production to loading area, using forklift to fork and load up onto lorry chassis, transport and deliver to scrapyard for disposal

Machine Spec

1 unit Waida Jig Grinding

Dim: 225 x 190 x 270 cm

Wt: 3, 500 kg

1 unit Matsuura Horizontal Mill

Dim: 360 x 240 x 250 cm

Wt: 8, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/06/25

Time of Operation

09:45 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solutions Asia Pacific

Equipment Use

2-3 x lorry loader

1 x 10ton forklift

Hydraulic/hand jack

Rollers/pallet truck

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

- Deploy lorry loader at unloading area

- Unload necessary equipment for the moving job

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Using 10ton forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175