# METHOD OF STATEMENT

Date: 16th Apr 2021

Description of work

To unload 3 cases from 1 x 40ft GP container, uncrate, hoist up onto lorry chassis, reverse lorry into factory premise, using factory overhead crane to unload machine

Machine Spec

3 cases of 1 unit XO 107-2000 PRE CMM machine

Case 1: 340 x 220 x 130 cm / Wt: 5, 000 kg

Case 2: 266 x 153 x 92 cm / Wt: 300 kg

Case 3: 140 x 110 x 103 cm / Wt: 227 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/04/21

Time of Operation

10:00 am

Location of Operation

#### 16 Benoi Rd

ST Engineering Marine Ltd

Equipment Use

1-2 x 633 lorry loader

Factory overhead crane

Wire/webbing sling

Shackle

Rollers/pallet truck

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deploy lorry loader at designated location (back to back with container)

- Pull out case from container onto lorry chassis

- Hoist down case to the ground

- Uncrate case

- Hoist up and detach machine from base

- Load machine onto lorry chassis

- Reverse lorry into factory production area

- Using factory overhead crane to unload machine from lorry chassis

- Shift and unload machine near clean room

- Using pallet truck to shift into clean room

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175