# METHOD OF STATEMENT

Date: 01st Oct 2024

Description of work

To unload 1 unit machine with parts from 1 x 20ft GP container, transport lorry into factory premise, using lorry and factory overhead crane to upright machine

Machine Spec

CJK5116E CNC Vertical Lathe

Dim 1: 3460 x 2150 x 1910 mm / Wt: 8 ton

Dim 2: 1580 x 880 x 800 mm / Wt: 0.5 ton

Dim 3: 1400 x 1500 x 1850 mm / Wt: 5.3 ton

Dim 4: 2100 x 630 x 500 mm / Wt: 1 ton

Dim 5: 1700 x 1700 x 800 mm / Wt: 0.2 ton

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

14-15/10/24

Time of Operation

09:45 am

Location of Operation

#### 7-9 Tuas Loop

Flowserve

Equipment Use

1 x 633 lorry loader

Factory overhead crane

Wire/webbing sling

Shackle

Hand jack

Rollers/pallet truck

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deploy lorry loader at designated location (back to back with container)

- Pull out case from container onto lorry chassis

- Uncrate case

- Transport lorry with machine into factory production area

- Using lorry and factory overhead crane to hoist and upright machine

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175