# METHOD OF STATEMENT

Date: 04th Oct 2024

Description of Work

To shift out 1 unit machine from one building to another building using forklift

Machine Spec

1 unit 7.10.7 CMM

Wt: 1, 260 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/10/24

Time of Operation

09:45 am

Location of Operation

#### GE Aviation Engine Services

Move out: 4 Seletar Link

Move in: 6 Seletar Link

Equipment Use

1 x lorry loader

1 x 3ton battery forklift

Crowbar

Woodblock

Pallet truck

Canvas/PVS sheet (to lay out for floor protection)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

- Deploy lorry loader at unloading area

- Hoist down forklift from lorry chassis

- Using forklift to fork up machine from inside factory

- Transport to another building

- Unload machine at door entrance

- Insert pallet truck under machine stand

- Shift and transport machine into production area

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175