# METHOD OF STATEMENT

Date: 04th Mar 2024

Description of Work

To shift out 1 unit machine from production to loading area, using forklift to fork up and transfer to new location, unload, shift and position into production

Machine Spec

1 unit Matsuura H-Plus 300

Pallet Changer & Chip Conveyor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20/03/24 - to shift out machine and transfer to new location using 10ton forklift

27/03/24 - to shift out pallet changer & chip conveyor to new location using factory pathway

Time of Operation

10:00 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solutions Asia Pacific

Equipment Use

1 x lorry loader

1 x 10ton forklift

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Canvas/steel plate (to lay out for floor protection)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

- Deploy lorry loader at unloading area

- Unload necessary equipment for the moving job

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Using 10ton forklift to fork up machine

- Transport machine to another building

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175