# METHOD OF STATEMENT

Date: 12th Sep 2023

Description of Work

1) To relocate 2 unit machine within factory premise and shift back upon machine move out/in

2) To shift out 1 unit machine from production to loading area, using forklift to fork up and transfer to outside carpark area and cover up with canvas

3) To shift out 1 unit machine from carpark to loading area, shift and position into factory production

Machine Spec

Relocate within factory premise

1 unit Hitachi Seiki CM40 lll

1 unit Hitachi Seiki VS40

To shift out from production to carpark area

1 unit Matsuura MC760V

To shift into production from carpark area

1 unit Matsuura V-Plus 800

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/09/23

Time of Operation

09:45 am

Location of Operation

#### 7 Joo Koon Circle

Honeywell (S) Pte Ltd

Equipment Use

1 x lorry loader

1 x low bed

1 x 10ton forklift

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Canvas/steel plate (to lay out for floor protection)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

- Deploy lorry loader at unloading area

- Unload necessary equipment for the moving job

- To jack up machine and insert rollers/pallet truck under machine bed

- To temporary relocate 2 unit machine about 1m away within production

- To shift out machine 1 unit machine from workshop to loading area

- Using 10ton forklift to fork up machine

- Transport machine to outside carpark area

- Lower down machine onto woodblock and cover up with canvas

- Using 10ton forklift to fork up another machine from carpark area

- Transport to loading area

- Insert rollers under machine bed

- Transport machine into production area

- Position machine at the designated location

- To relocate back 2 unit machine to the original location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175