# METHOD OF STATEMENT

Date: 11th July 2024

Description of Work

To relocate following machine within factory premise

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

To shift within building A

2 unit Balancing machine

1 unit 3M Lathe

To shift from building A to building B

1 unit Milling

1 unit Bandsaw

1 unit Radial Drill

To shift from centre building to building A using forklift

1 unit Furnace

Time of Operation

09:30 am

Location of Operation

#### 29 Tuas Ave 2

Hilco Construction

Equipment Use

1 x lorry loader

1 x 4.5ton forklift

Hydraulic/hand jack

Woodblock

Rollers

Pallet truck

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

# To shift within building A

- Deploy lorry loader at unloading area

- Unload necessary equipment for the moving job

- Jack up machine to 3” high

- Insert rollers/pallet truck under machine bed

- Shift out from existing to new location within premise

- Position at designated location

- Unload from rollers and insert levelling pad under machine bed

To shift out from building A to building B

- Jack up machine to 3” high

- Insert rollers/pallet truck under machine bed

- Shift out machine from production to loading area

- Transport machine to another building

- Position machine at the designated new location

- Unload from rollers and insert levelling pad under machine bed

To shift from centre building to building A using forklift

- Using 4.5ton forklift to fork up furnace

- Transport furnace to building A

- Shift furnace into production area

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175