# METHOD OF STATEMENT

Date: 21st Mar 2023

Description of work

To hoist down machines and accessories from 3rd floor using platform and mobile/lorry crane, load up onto lorry chassis and deliver to new factory

Machine Spec

1 unit YCM NXV1020 CNC

Dim

1 unit Bandsaw

1 unit Compressor

1 unit Cutting machine

2 unit Milling machine

1 lot table

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

/23

Time of Operation

09:45 am

Location of Operation

#### 14 A.M.K Industrial Pk 2 #03-01

Common Alliance Engineering Pte Ltd

Equipment Use

1 x 50ton mobile crane

1 x 633 lorry loader

2 x lorry

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Mobile crane to hoist up platform to 3rd floor roller shutter opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift machine into platform

- Insert sling to crane hook and secured to rigging point of machine

- Mobile crane to hoist down machine to the ground

- Load machine onto lorry chassis

- Secured with strapping belt prior to moving out of premise

- Transport to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175