# METHOD OF STATEMENT

Date: 05th July 2019

Description of work

To shift out 4 unit machine from production room to loading area, hoist up and load onto wooden base, after packing load onto 2 x 40ft HQ & 2 x 40ft FR container

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/07/2019   -   To shift out 1 x Aida 80ton Press & Aida 110ton Press

11/07/2019   -   To shift out 1 x Aida NCS 200ton Press & Patek PHL-200K Press

Time of Operation

09:30 am

Location of Operation

#### No 1 Bedok South Rd

Panasonic Refrigeration Devices (S) Pte Ltd

Equipment Use

1 x 100 ton mobile crane

1 x 50 ton lorry crane

1 x forklift

1 x scissor lifter

Heavy duty rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to designated packing area

- Hoist down machine from lorry and load onto wooden base for packing

- After packing, hoist up and load case onto customer low loader

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175