# METHOD OF STATEMENT

Date: 08th Feb 2017

Description of work

To shift out 1 unit machine from production room to loading area, hoist up and load onto Malaysia low-bed

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

TBA

Location of Operation

#### 1 Changi North St 1

Frencken ETLA Limited

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, if any will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert rollers/pallet truck to machine base

- Shift out machine from production room to loading bay

- Hoist up and load machine onto Malaysia low-bed

- Secured machine with belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping,

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175