# METHOD OF STATEMENT

Date: 26th Feb 2024

Description of work

To unload and hoist down 7 cases of 1 unit machine with accessories from low-bed to the ground, uncrate, shift and position into factory premise at ground

Machine Spec

Case No.1 3.480m x 3.340m x 1.400m

Case No.2 3.480m x 3.340m x 1.400m

Case No.3 3.480m x 3.340m x 1.660m

Case No.4 3.230m x 1.930m x 3.410m

Case No.5 1.900m x 1.000m x 2.470m

Case No.6 4.000m x 2.000m x 1.500m

Case No.7 2.090m x 1.020m x 0.660m

Case No.1/1 e 300kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/03/24

Time of Operation

11:30 am

Location of Operation

#### 10 Riverside Rd

Okamoto (S) Pte Ltd

Equipment Use

633 lorry loader

Low-bed

Rollers

Pallet truck

Wire/webbing sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated unloading area

- Hoist up case from FR container and unload to the ground

- Unscrew all bolt & nuts from steel case

- Attached sling to crane hook and secured onto case

- Hoist up case above machine

- Swing case away from machine and lower down to the ground

- Release all sling from case

- Rotate crane towards machine

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck onto machine base

- Transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from roller and insert levelling plates under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175