# METHOD OF STATEMENT

Date: 15th July 2024

Description of work

To unload 3 skid of 1 unit machine from 1 x 40ft HQ container to the ground, shift and position into factory premise at ground

Machine Spec

3 packages of 1 unit Yamasaki Giken YMZ-850

Skid 1: 328 x 228 x 255 cm / Wt: 5, 050 kg

Skid 2: 220 x 130 x 165 cm / Wt: 200 kg

Carton 3: 36 x 25 x 20 cm / Wt: 25 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

18/07/24

Time of Operation

09:30 am

Location of Operation

#### 10 Riverside Rd

Okamoto (S) Pte Ltd

Equipment Use

633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry crane back to back with container

- Using crane to pull out skid from container onto lorry chassis

- Attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck onto machine base

- Transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from roller and insert levelling plates under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175