# METHOD OF STATEMENT

Date: 17th Mar 2023

Description of work

To unload and hoist down 4 cases of 1 unit machine with accessories from 2 x 40ft FR and 1 x 20ft GP container to the ground, uncrate, shift and position into factory premise at ground

Machine Spec

1 unit Mazak HCN-8800

Dim: 7, 230 x 3, 530 x 3, 680 mm

Wt: 28, 780 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/03/23

Time of Operation

10:00 am

Location of Operation

#### 10 Riverside Rd

Okamoto (S) Pte Ltd

Equipment Use

1 x 70ton mobile crane

633 lorry loader

1 x forklift

1 set rollers

Pallet truck

Wire/webbing sling

Hand drill

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile crane at designated unloading area

- Hoist up case from FR container and unload to the ground

- Unscrew all bolt & nuts from steel case

- Attached sling to crane hook and secured onto case

- Hoist up case above machine

- Swing case away from machine and lower down to the ground

- Release all sling from case

- Rotate crane towards machine

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers under machine bed and secure with bolt and nuts

- Transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from roller and insert clamping plates under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Mobile: 92381431