# METHOD OF STATEMENT

Date: 06th Mar 2019

Description of work

1) To shift out and hoist down 6 unit machine from 2nd floor through door opening by using platform, mobile/lorry crane and load machine onto Malaysia lorry/low bed

2) To shift out small machine and equipment from 2nd floor via cargo lift to loading bay and load onto Malaysia lorry

Machine Spec

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11/03/2019

Time of Operation

09:30 am

Location of Operation

#### 10 Tampines North Drive 4 #02-01, JTC Space @ Tampines North

Bloomfoss Pte Ltd

Equipment Use

1 x 50 tom mobile crane

1 x 50 ton lorry crane

1 x platform

1 set wire sling

1 set shackle

1 x platform

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area

- Hoist up platform to 2nd floor door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secured front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load into platform

- 50 ton mobile crane to hoist down machine and load onto Malaysia low bed/lorry

- Secured machine with strapping belt prior to moving out from premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175