# METHOD OF STATEMENT

Date: 29th Oct 2020

Description of work

To shift out and hoist down 5 set of U15 machine and 2 set of machine line from 2nd floor through shutter door opening by using lorry crane

Machine Spec

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Time of Operation

Location of Operation

#### 10 Tampines North Drive 4 #01-03, JTC Space @ Tampines North

Equipment Use

1 x 633 lorry loader

2/3 x lorry loader

1 set wire sling

1 set webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Shift out machine from production to near shutter door opening

- Crane operator will rotate crane and lowering boom until it reach machine location

- Riggers will attached required lifting gears to crane hook and secured onto lifting point of machine to be lifted

- Signalman will give signal to crane operator to hoist up machine slowly to check on balancing

- Secured machine with strapping belt prior to moving out from premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175