# METHOD OF STATEMENT

Date: 15th Mar 2018

Description of Work

To unload 3 unit machine with accessories from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

3 unit YCM NXV1020A CNC

Dim: 340 x 227 x 257 cm

Wt: 6, 500 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

26/03/2018   -   09:30 am

Location of Operation

#### 10 Tampines North Drive 4 - JTC Space @ Tampines North

SMT Technology Pte Ltd #01-04

Equipment Use

2-3 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc.. )

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Hoist down case from lorry chassis

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert rollers/pallet truck to machine base

- Shift and transport machine into premise at ground floor

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175