# METHOD OF STATEMENT

Date: 08th Oct 2024

Description of work

To hoist up 1 unit machine to 3rd floor, lower down onto open space balcony, shift and position into customer premise

Machine Spec

1 unit Litz SV1100 CNC

Dim: 2.95m x 3.4m

Wt: 9, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/10/24 (Mon)

Time of Operation

Estimate:

13:00 pm

Location of Operation

#### 10 Tampines North Drive 4 #03-04  –   JTC Space @ Tampines North

SK Global Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x lorry

1 x low-bed

Rollers/pallet truck

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited to by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area for the lift

- Setting up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Reverse low-bed to near crane

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Mobile crane to hoist up machine 1ft above ground to check on balancing

- After well balance, hoist up machine to 3rd  floor

- Lower down machine at 3rd floor open space balcony

- Insert rollers under machine bed

- Shift and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175