# METHOD OF STATEMENT

Date: 13th Mar 2023

Description of work

To hoist down 1 unit machine from 3rd floor through open space balcony and load up onto lorry chassis

Machine Spec

1 unit VM10i CNC

Dim: 1.8m x 1.74m x 2.3m

Wt: 3, 300 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/03/23

Time of Operation

13:00 pm

Location of Operation

#### 10 Tampines North Drive 4 #03-10   –   JTC Space @ Tampines North

Guoxin Manufacturing Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

Safety Aspect

Implementation of traffic control such as assignment of worker to act as banksman/traffic controller to assist in controlling movement of pedestrians and vehicle within the site vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting area for the lift

- Setting up mobile crane at site location

- Shift out machine to open space balcony

- Insert sling to crane hook and secure to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- After well balance, hoist down machine from 3rd  floor to the ground

- Load up machine onto lorry chassis

- Release all sling from machine rigging point

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175