# METHOD OF STATEMENT

Date: 27th Dec 2017

Description of work

To hoist up 1 unit machine to 2nd floor through shutter door opening by using lorry crane

Machine Spec

1 unit CG Grinder

Dim: 313 x 228 x 220 cm

Wt: 3, 360 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

29/12/2017   -   12:00 pm

Location of Operation

#### 10 Tampines North Drive 4 #02-01, JTC Space @ Tampines North

Bloomfoss Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area

- Attached wire/webbing sling to crane hook and insert onto machine hoisting point

- Hoist up machine to 2nd floor through shutter door opening

- Lower down machine slowly to near ground level

- Insert pallet truck to machine base

- Shift and position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175