# METHOD OF STATEMENT

Date: 24th May 2023

Description of Work

1) To shift out 2 unit machine from production to loading area, using forklift to fork up onto lorry chassis

2) To unload 1 unit machine from lorry chassis, shift and position into production

Machine Spec

Move out

2 unit VC500i

Dim: 280 x 230 x 255 cm

Wt: 7, 170 kg

Move in

1 unit BX40Ui

Dim: 330 x 230 x 255 cm

Wt: 8, 200 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/05/23

Time of Operation

09:45 am

Location of Operation

#### 79 Loyang Way

#### Fong Engineering

Equipment Use

2-3 x lorry loader

1 x 10 ton forklift

Rollers/pallet truck

Canvas, metal sheet (to lay out for floor protection)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

- Jack up machine to 3” high using hydraulic/hand jack

- Insert rollers to machine bed

- To shift out machine from production to loading area

- Using 10ton forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- To unload and fork down 1 unit machine from lorry chassis

- Transport to unloading area

- Insert rollers under machine bed

- Shift and transport into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175