# METHOD OF STATEMENT

Date: 27th Sep 2022

Description of Work

To shift out 3 unit machine from production to loading area, using forklift to fork up onto wooden base for packing, after pack load onto 2 x HQ container and secured

Machine Spec

Makino F3 machine

Dim: 300 x 210 x 235 cm

Wt: 7, 300 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/10/22 (Mon)

Time of Operation

09:45 am

Location of Operation

#### Blk 2 Kaki Bukit Ave 1 #01-02

Micro-Mechanics Pte Ltd

Equipment Use

1 x lorry loader

1 x low bed

Rollers/pallet truck

1 x 10 ton forklift

Packing materials

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation (attached all lifting crew certificate)

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# Job scope

- To shift out machine from production to loading area

- 10ton forklift to fork up machine

- Load up machine onto wooden base

- Pack to form a case

- After packing, load up case onto container

- Secured case with choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175