# METHOD OF STATEMENT

Date: 10th Aug 2022

Description of Work

To shift out 1 unit machine from workshop to loading area, using forklift to fork up onto lorry chassis and deliver to another location, unload, shift and position into premise at ground floor

Machine Spec

1 unit Phoenix V/Tome Xray machine

Dim: 260 x 180 x 210 cm

Wt: 8, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/08/22 (Monday)

Time of Operation

09:45 am

Location of Operation

#### Collection: 73 Nanyang Drive, Simtech Valley Block

Deliver: 3 Cleantech Loop, JTC Cleantech II

Equipment Use

1 x lorry loader

1 x low bed

Rollers/pallet truck

1 x 10 ton forklift

Canvas/steel plate (to lay out for floor protection at both site)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

- To shift out machine from workshop to loading area

- 10ton forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to new location

- Using 10ton forklift to fork down machine from lorry chassis

- Unload machine to near ground level

- Insert rollers/pallet truck to machine base

- Transport machine into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175