# METHOD OF STATEMENT

Date: 16th Apr 2024

Description of Work

To unload 2 case of 1 unit machine from low bed using forklift, uncrate, shift and position into tenant premise

Machine Spec

INT J200 machine

Case 1: 475 x 271 x 292 cm

N/G Wt: 7, 910 kg / 9, 150 kg

Case 2: 348 x 100 x 199 cm

N/G Wt: 370 kg / 670 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/04/24

Time of Operation

09:45 am

Location of Operation

#### 6 Yishun Industrial St 1, North View Bizhub

Crager Industries Pte Ltd #03-17

Equipment Use

1-2 x lorry crane

Rollers/pallet truck

Hydraulic/hand jack

1 x 10 ton forklift

1 x low bed

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy low loader at designated unloading area

- Unload forklift from low loader

- Using forklift to fork down case from lorry chassis

- Unload case to the ground

- Uncrate case and put aside

- Use forklift to detach machine from base

- Transport machine to unloading area

- Lower down machine to near ground level

- Insert roller under machine bed

- Transport machine into tenant premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175