# METHOD OF STATEMENT

Date: 25th Jan 2019

Description of work

To shift out 3 unit machine from production area to loading bay, lay down, hoist up onto

low loader and transport to another location, unload, shift and position into premise at ground floor

Machine Spec

2 unit Aida 200 ton Power Press

1 unit Aida 110 ton Power Press

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/01/2019   –   10:00 am

Location of Operation

#### From 11 Tuas Ave   –   3 Pioneer Sector Walk

Asia Pacific Spiral

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Lorry/low loader

Rollers/pallet truck

1 set wire sling/shackle

1 x forklift

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine

- Insert rollers to machine base

- Shift out machine from production room to loading bay

- Hoist and lay down machine

- Load machine onto low loader

- Secured machine with strapping belt prior to moving out of premise to new location

- At new premise, hoist down machines from low-loader

- Upright machine

- Insert rollers to machine base

- Shift and transport machine into factory

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175