# METHOD OF STATEMENT

Date: 16th Dec 2019

Description of Work

To unload 12 cases of 1 set Gundrill machine from container, uncrate, hoist and detach machine from base, shift and position into production at ground floor

Machine Spec

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

18-20/12/2019

Time of Operation

10:00 am

Location of Operation

#### 11 Tuas South Ave 12

Halliburton Completions MFG Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK100002E

Air rollers/pallet truck

Wire/webbing sling

Canvas, plywood (for flooring protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down machine case from container to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers to machine base for the move

- Shift and transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175