# METHOD OF STATEMENT

Date: 24th July 2018

Description of work

To shift out 9 unit machine from production area to loading bay @ old factory, load onto lorry chassis, secured machine with strapping belt and deliver to new factory

1)

Unload, shift & position to 5th floor via cargo lift

1 unit SHP2

1 unit SPH1

1 unit E50

2 unit NAF 470

1 unit SPA 750

2)

Hoist up to 5th floor using platform & mobile crane

1 unit Comelz XL, CZ-XL

1 unit NEK

1 unit Hydraulic Hot Tapping

1 unit Toggle machine – to unload from forwarder lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

03/08/2018   -   09:00 am

Location of Operation

#### From 12 Defu Lane 8   -   6 Kim Chuan Rd

Heng Long Leather Co. Pte Ltd

Equipment Use

2 x 50 ton mobile crane

2-3 x lorry crane

1 x platform

1 x forklift

Pallet truck/rollers

Shackle

Wire/webbing sling

Obstacles

No obstacle on loading/unloading bay and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 2 x 50 ton mobile crane to park at designated hoisting area for the lift

- 1 x mobile crane to hoist up platform to 5th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 1 x 50 ton mobile crane

- To drill and fix eyebolt/low plug onto building pillar or flooring – to support and securing of strapping belt onto platform to prevent platform from swinging outward

- 50 ton mobile crane to hoist up machine to 5th floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck onto machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175