# METHOD OF STATEMENT

Date: 04th Oct 2018

Description of work

To shift out 2 unit machine from production room to loading area, hoist up and load onto lorry chassis and deliver to new location

Machine Spec

1 x HL25 CNC Lathe

Dim: 255 x 230 x 200 cm / Wt: 4, 000 kgs

1 x HL55 CNC Lathe

Dim: 565 x 230 x 225 cm / Wt: 8, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

08/10/2018   –   09:45 am

Location of Operation

#### 12 Tuas Ave 2

Flowserve Pte Ltd

Equipment Use

2 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers/pallet truck to machine base

- Shift out machine from production area to loading bay

- Lorry crane to hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175