# METHOD OF STATEMENT

Date: 03rd June 2022

Description of work

To shift out following machine from outside shelter to loading area, hoist up onto low loader chassis and deliver to Eric Wong warehouse

Machine Spec

2 unit Mori SL-603/2000

Dim: 610 x 285 x 290 cm

Wt: 16, 000 kg

1 unit Mori SL803C

DIM: 550 x 230 x 240 cm

Wt: 16, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Schedule of Operation

10/06/22

Time of Operation

09:45 am

Location of Operation

#### Hunting Energy Services Pte Ltd

16E Tuas Ave 1

Equipment Use

1 x 633 lorry loader

Lorry loader

Low loader

Rollers/pallet truck

Hydraulic/hand jack

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area and route of moving operation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involve in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine to hoisting area

- Attached required lifting gears to crane hook and secured to hoisting rigging point of load to be lifted

- Lorry crane to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto low loader chassis

- Release all sling from machine hoisting point

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to Eric Wong warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175