# METHOD OF STATEMENT

Date: 21st Dec 2022

Description of Work

To shift out 5 unit machine and accessories from production to loading area, using forklift to fork up onto wooden base for packing, after packing load up onto low-bed

Machine Spec

1 unit Doosan Puma 2500 LSY CNC Lathe

Dim: 340 x 185 x 208 cm / Wt: 5, 400 kg

1unit Doosan Puma 300LM CNC Lathe

Dim: 400 x 205 x 200 cm / 6, 700 kg

1 unit Doosan Puma 400L CNC Lathe

Dim: 510 x 215 x 230 cm / Wt: 9, 800 kg

1 unit Doosan Puma 480xlm CNC Lathe

Dim: 670 x 240 x 245 cm / Wt: 12, 500 kg

1 unit Doosan Puma MYNX 540/50 CNC Milling

Dim: 320 x 310 x 310 cm / Wt: 7, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/01/23 – 06/01/23

Time of Operation

09:45 am

Location of Operation

#### 7 Loyang Way

#### NOV Oilwell Varco

Equipment Use

Lorry loader

1 x low bed

Rollers/pallet truck

1 x 16 ton forklift

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.OM certified forklift operator will be involved in the operation

# Job scope

- Jack up machine to 3” high using hydraulic/hand jack

- Insert rollers to machine bed

- To shift out machine from production to loading area

- Using 16ton forklift to fork up machine

- Load machine onto wooden base

- Pack to form a case

- After packing, load up case onto customer arranged low-bed

- Secured machine with strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175