# METHOD OF STATEMENT

Date: 09th Nov 2018

Description of work

To shift out 1 unit OKUMA MA-50HB and 1 unit BROWN & SHARP CMM machine from production area to loading bay, hoist up onto low-loader/lorry chassis and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

09-12/11/2018   -   09:30

Location of Operation

#### 18 Bedok South Rd

UTC Aerospace System

Equipment Use

1 x 70 ton mobile crane

1 x 50 ton lorry crane

1 x low-loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Metal sheet/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hook wire/webbing sling onto machine hoisting point

- Mobile crane to hoist up machine to about 6” from the ground

- If stability and balancing is not achieved, lifting supervisor will stop the hoisting and riggerman will re-adjust the sling or adding shackle till stability is achieved

- Once stability is achieved, hoist up machine and load onto low-loader

- Release all sling from machine hoisting point

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver  machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175