# METHOD OF STATEMENT

Date: 14th Aug 2025

Description of work

To shift out 1 unit OKUMA MA-50HB and 1 unit BROWN & SHARP CMM machine from production area to loading bay, hoist up onto low-loader/lorry chassis and deliver to new location

Machine Spec

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

10:00 am

Location of Operation

#### 18 Bedok South Rd

UTC Aerospace System

Equipment Use

1 x 633 lorry loader

1 x low-loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Hydraulic/hand jack

Metal sheet/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production to hoisting bay

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Mobile crane to hoist up machine to about 1ft above ground

- If stability and balancing is not achieved, lifting supervisor will stop the hoisting and riggerman will re-adjust the sling or adding shackle till stability is achieved

- Once stability is achieved, hoist up machine and load onto low-loader

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175