# METHOD OF STATEMENT

Date: 07th Mar 2018

Description of Work

To unload 1 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit Mitsubishi NC-EDM

Dim: 285 x 229 x 232 cm

Wt: 2, 300 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

15/03/2018   -   10:00 am

Location of Operation

#### No 18 Tractor Rd

NOK Precision Component Singapore Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (certificate shall be available at all time)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, cotton gloves)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

Certificate shall be available at all times

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Hoist down case from lorry chassis

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert pallet truck to machine base

- Transport machine into premise

- Position machine to the designated location

- Unload machine from pallet truck and let it sit on machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175