# METHOD OF STATEMENT

Date: 14th May 2018

Description of work

To shift out and hoist down 1 set Vacuum Coater from 4th floor by using platform and mobile/lorry crane for packing, after pack, stuff into 1 x 40ft OT & 1 x 20ft GP container

Machine Spec

1 set of Shincron CES-3Q-H1 Vacuum Coater

Main Coater Dim: 410 x 175 x 265 cm

Wt: 4, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

19/05/2018   -   09:30 am

Location of Operation

#### No 18 Woodlands Loop #04-00

Edmund Optics Singapore Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

Wire/webbing sling

Shackle

1 x platform

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area

- Hoist up platform to 4th floor door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- Shift out machine from production and load into platform

- 50 ton mobile crane to hoist down machine to ground floor

- Load machine onto wooden for packing

- After packing, stuff case into container

- Secured machine with choking wood and strapping/lashing belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175