# METHOD OF STATEMENT

Date: 18th Apr 2017

Description of Work

To unload 2 unit machine from 1 x 40ft HQ container, uncrate, shift and position into customer premise at level 1

Machine Spec

Roders RXP 400 DS

Dim: 2515 x 1930 x 2070 mm

Wt: 3, 200 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 25/04/2017   -   10:00 am

Location of Operation

#### No 2 A.M.K Drive – Blk G / Level 1

ITE College Central

Equipment Use

1 x 50 ton lorry crane

Pallet truck/rollers

1 x platform (to lay out for machine passage way)

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location (back to back with container)

- Pull out machine case from container onto lorry chassis

- Uncrate machine case

- Hoist and detach machine from base

- Load machine onto platform

- Insert pallet truck/rollers to machine base for the move

- Shift and position machine to the designated location at level 1

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175