# METHOD OF STATEMENT

Date: 11th July 2017

Description of Work

To unload 2 cases of equipment from lorry chassis, uncrate, hoist and detach equipment from base, shift and position into customer premise at ground floor

Equipment Spec

1 unit Roll Feeder

Dim: 107 x 107 x 71 cm

Wt: 90 kgs

1 unit Uncoiler

Dim: 130 x 92   x 130 cm

Wt: 230 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 18/07/2017

Location of Operation

#### 2 A.M.K Drive

ITE College Central

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area

- Uncrate case

- Hoist up and detach equipment from base

- Swing crane to loading area

- Insert pallet truck to equipment base for the move

- Shift and position equipment to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175