# METHOD OF STATEMENT

Date: 23rd Aug 2017

Description of work

To unload 10 unit machine from lorry/low-loader chassis, uncrate, hoist up to 5th floor by using platform and mobile crane

Machine Spec

CTX 310 CNC Lathe machine

Dim: 290 x 160 x 175 cm

Wt: 4, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

30/08/2017

Time Of Operation

09:30 am

Location of Operation

#### 2 AMK Drive

ITE College Central

Equipment Use

2 x 50 ton mobile crane

Lorry crane

1 x low loader

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Clear all obstacles in way of lifting path

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and signage                                                             All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 5th floor opening

- Tied back platform to a competent point and secured using 50 ton mobile crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 5th floor

- Lower down and load machine onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform

- Position machine to designated location at tool room

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175