# METHOD OF STATEMENT

Date: 21st Aug 2020

Description of Work

To unload 1 unit machine and parts from low-loader/lorry chassis, shift into factory premise at ground floor, to assist customer engineer assemble machine by using factory overhead crane

Machine Spec

1 unit G & LVTC 200

Column: 280 x 130 x 370 cm / Wt: 14, 000 kg

Oil Tank: 200 x 100 x 180 cm

Chiller: 140 x 140 x 180 cm

Control: 200 x 100 x 220 cm

Base: 500 x 280 x 100 cm / Wt: 18, 800 kg

Cross Beam: 500 x 120 x 100 cm / Wt: 3, 000 kg

Spindle Head: 450 x 100 x 100 cm / Wt: 2, 000 kg

Chip Conveyor: 600 x 80 x 280 cm

ATC: 200 x 200 x 200 cm

Scissor working platform: 300 x 240 x 100 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 2 Gul Circle

#### Cooper Cameron (S) Pte Ltd

Equipment Use

1 x 70 ton mobile crane

2 x 633 lorry loader

1 x boom lift

1 x 20 ton factory overhead crane

1-2 x low-loader

1 set wire sling (SWL 4ton)

1 set wire sling (SWL 8ton)

Shackle (SWL 6.5 ton, SWL 8.5ton, SWL 12ton)

1 x forklift

Rollers/pallet truck

Canvas/metal plate (for epoxy floor protection)

Obstacles

To clear all obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting location for the lift

- Reverse low-loader to near crane

- Riggers to attached all required lifting gears and secure to rigging point of load to be lifted

- Hoist up machine/parts from low bed

- Move low bed away

- Lower down machine/part to the ground

- Insert rollers under machine/parts base

- Shift and position into factory premise

- Position machine base at the designated location

- Using factory overhead crane to assist customer engineer to install machine by parts

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175