# METHOD OF STATEMENT

Date: 03rd March 2017

Description of work

To hoist up 1 set of Packaging machine to 6th floor by using platform and mobile crane

Machine Spec

Case 1: 559 x 198 x 221 cm / Wt: 2042 kgs

Case 2: 417 x 178 x 211 cm / Wt: 1361 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

07/03/2017   -   10:00 am

Location of Operation

#### 2 Chia Ping Rd

Haw Par Healthcare Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Scissor lifter

Wire sling

Metal plate (for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc…safety shoes, helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile crane to park at designated location for the hoist

- Load machine onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist up platform together with machine to 6th level opening

- Stabilize platform effectively and ensure it is fully rested on the parapet wall

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert pallet truck to machine base

- Shift out machine from platform and load onto scissor lifter

- Lower down scissor lifter, shift out machine, insert pallet truck/rollers and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175