# METHOD OF STATEMENT

Date: 28th Dec 2020

Description of Work

To unload 5 packages of chip conveyor from 1 x 40ft HQ container, uncrate cases, shift and position into customer premise at ground floor

Machine Spec

5 packages of 2 unit Ohimine Chip Conveyor

Crate 1: 718 x 107 x 140 cm / Wt: 660 kg

Crate 2: 437 x 85 x 135 cm / Wt: 150 kg

Crate 3: 332 x 85 x 83 cm / Wt: 130 kg

Crate 4: 309 x 153 x 96 cm / Wt: 260 kg

Crate 5: 202 x 108 x 94 cm / Wt: 470 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29-31/12/20

Time of Operation

09:45 am

Location of Operation

#### 20 Benoi Sector

Shimano Singapore Pte Ltd

Equipment Use

1 x 633 lorry crane

Pallet truck / rollers

Wire/webbing sling

1 x forklift

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location (back to back with container)

- Crane to pull out crate from container onto lorry chassis

- Hoist down and unload crate to the ground

- Hoist up and detach conveyor from base

- Swing crane to loading bay

- Insert rollers/pallet truck under conveyor bed

- Shift and transport into production at ground floor

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175