# METHOD OF STATEMENT

Date: 08th Dec 2022

Description of work

To transport 4 item of 1 set machine using low-bed, using forklift to unload at exit point, shift and position into customer premise at ground floor

Machine Spec

1 unit Kuraki Boring machine

1: 457 x 308 x 322 cm / Wt: 14, 000 kgs

2: 303 x 196 x 151 cm / Wt: 2, 500 kgs

3: 353 x 227 x 188 cm / Wt: 2, 500 kgs

4: 369 x 227 x 216 cm / Wt: 3, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/12/22 (Saturday)

Time of operation

9:45 am

Location of Operation

#### 20 Bukit Batok Crescent #01-12

Hui Hong

Equipment Use

1-2 x 633 lorry loader

1 x low-bed

1 x 16 ton forklift

Rollers/pallet truck

Metal plate (to lay out for floor protection)

Obstacles

No obstacle around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator will be involved in the operation

# JOB SCOPE

- Deploy low-bed with machine at exit point of unloading area

- Using 16ton forklift to fork down machine from low-bed

- Transport machine to tenant door entrance

- Lower down machine to near ground level

- Insert roller under machine base

- Shift and transport machine into production area

- Position machine to the designated location at ground floor

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175