# METHOD OF STATEMENT

Date: 06th Aug 2018

Description of work

To shift out machineries from customer production room at ground floor, hoist up onto lorry chassis and deliver to new factory

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

06/08/2018   -    09:45 am

Location of Operation

#### 21 Joo Koon Crescent   -   4 Tuas Drive 2

AVA Global Pte Ltd

Equipment Use

1 x 50 ton lorry crane

2/3 x lorry crane

1 x low loader

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Shift out machine from production room to loading area

- Hoist up and load machine onto low loader/lorry chassis

- Secured machine with strapping belt prior

- Transport and deliver machine to new factory

- At new factory, unload machine, shift and position into premise at ground floor

- To hoist up 5 unit machine to 2nd floor through shutter door opening, shift and position to location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175