# METHOD OF STATEMENT

Date: 20th May 2020

Description of Work

1) To shift out 2 case of 1 unit new granite table from customer warehouse, uncrate shift and position into production

2) To dismantle existing CMM bridge, granite table and replace with new granite table, install bridge, shift and position into CMM room at ground floor

New Granite Spec

Dim: 330 x 230 x 85 cm

G/Wt: 7, 700 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/06/2020

Time of Operation

09:30 am

Location of Operation

#### No 21 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x lorry crane

1 x 3ton battery forklift

Customer overhead crane

Pallet truck/rollers

Webbing sling

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out case of new granite table from warehouse to outside

- Uncrate case

- Shift granite into factory

- To shift out existing CMM to near overhead crane

- Using overhead crane to dismantle CMM bridge, granite table and put aside

- Lift up new granite table and load onto stand

- Install CMM bridge

- After installing is done, shift back into CMM room

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175