# METHOD OF STATEMENT

Date: 02nd May 2019

Description of work

To unload 1 unit machine from 1 x container, uncrate, shift and position into customer production area at ground floor

Machine Spec

1 unit New PWT Refurbished Acuweld System

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Location of Operation

#### 21 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK100002

Rollers/pallet truck

Wire/webbing sling

1 x 3 ton battery forklift

Metal plate/canvas (for floor protection)

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Pull out case from container and load onto lorry chassis

- Transport lorry to loading area

- Hoist down machine case from lorry chassis

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to loading area

- Insert rollers under machine base

- Shift machine into production area and position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175