# METHOD OF STATEMENT

Date: 27th Mar 2018

Description of Work

1) To unload 4 set DP-920G CE Variable Spindle Speed machine @ net weight of 360 kg each from lorry chassis, shift and position to 4th floor via cargo lift

2) To shift out 4 unit Small Drilling machine @ 220 kg each from 4th floor, load onto lorry chassis and deliver to customer office

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

29/03/2018   -   Afternoon schedule

Location of Operation

#### No 21 Tampines Ave 1

Temasek Polytechnic, Blk 25 #04-109

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for loading/unloading

- Hoist and lower down machine from lorry chassis onto the ground

- Insert pallet truck to machine base for the move

- Transport machine to 4th floor via cargo lift

- Shift and position machine to the designated location

- To shift out 4 unit drilling from 4th floor and load onto lorry chassis

- Transport and deliver machine to customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175