# METHOD OF STATEMENT

Date: 30th Oct 2020

Description of Work

To shift out 1 unit RIE machine, 1 unit Chiller and 1 unit Rough Pump from clean room at ground floor to loading area, hoist up and load onto lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/11/20

Time of Operation

09:30 am

Location of Operation

#### 21 Tampines Ave 1, Temasek Polytechnic

Blk 34 L1, EN34-1-28

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Plywood (to lay out for floor protection)

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading/unloading area

- Shift out machine from clean room at level 1 to loading area

- Insert sling onto machine hoisting point

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175