# METHOD OF STATEMENT

Date: 08th Aug 2019

Description of Work

1) To unload 2 cases from lorry chassis, uncrate, hoist and detach machine from base, shift and position into customer X-ray room at ground floor

2) To shift out 1 unit X-ray machine from X-ray room to loading bay, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

2 cases of X-ray machine

Case 1 Dim: 202 x 190 x 217 cm

Wt: 2, 606 kgs

Case 2 Dim: 105 x 135 x 72 cm

Wt: 86 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/08/2019

Time of Operation

10:00 am

Location of Operation

#### 249 Jalan Boon Lay

ST Kinetics

Equipment Use

1 x 633 lorry crane

Pallet truck

Wire/webbing sling

Canvas, plywood (for flooring protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Hoist down machine case

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to near door entrance

- Lower down machine to near ground level

- Insert pallet truck to machine base for the move

- Shift and transport machine into X-ray room at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175