# METHOD OF STATEMENT

Date: 12th Dec 2019

Description of Work

To unload 1 unit machine from lorry chassis, uncrate, shift and position into customer premise

Machine Spec

1 unit HVS-300 Vacuum Oil Degasifier

Dim: 180 x 160 x 210 cm

Wt: 2, 900 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/12/2019 (Sat)

Time of Operation

09:30 am

Location of Operation

#### No 25 A.M.K St 65

Sanden International (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck/rollers

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane and lower down machine from lorry chassis to near ground level

- Insert pallet truck to machine base for the move

- Transport machine into premise

- Shift and position machine at the designated location

- Unload machine from pallet truck and let it sit onto machine pad

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175