# METHOD OF STATEMENT

Date: 03rd Jan 2020

Description of work

To unload 4 wooden case of 1 set machine from 1 x 20ft FR and 1 x 20ft GP container using forklift, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit Yasda, Model: YCM430 Ver lll

Case 1: 313 x 216 x 322 cm / Wt: 7, 700 kgs

Case 2: 96 x 96 x 158 cm / Wt: 240 kgs

Case 3: 179 x 102 x 91 cm / Wt: 260 kgs

Case 4: 197 x 140 x 67 cm / Wt: 240 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

06/01/2020   –   10:00 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solution Asia Pacific

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 x 10 ton forklift

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones and red/white tape

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator will be involved in the operation

# JOB SCOPE

- 10 ton forklift to fork down cases from container

- Unload case to the ground

- Uncrate case

- Fork up and detach machine from base

- Transport machine to door entrance

- Lower down machine and insert roller to machine base

- Shift and transport machine into production area

- Position machine to the designated location at ground floor

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175