# METHOD OF STATEMENT

Date: 11th Sep 2017

Description of work

To shift out and hoist down 1 unit Mitsubishi Heavy Industries Precision CNC machine from 5th floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/09/2017   -   09:30 am

Location of Operation

#### Blk 5004 A.M.K Ave 5 #05-03/04

2M Precision Automation Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness,  etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 5th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production and load onto platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine

- Load platform and machine onto lorry chassis

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regard

Albert

Hup Hin Transport Co Pte Ltd

Tel: 62621525